	2WDRC.com

4WDRC.com
	X-6 SETUP SHEET
	X-FACTORY R/C

XFactoryRC.com

	Driver: Petri Ström
	Date: 11.9.2010

	Track/Location: Vaasa
	Event: Maastosarja 3

	 □Tight/   ■Open
	   □Wet/  ■Dry
	   ■Smooth/  □Rough
	Slippery □  □  □  ■  □ High-Bite

	FRONT

	SUSPENSION

RIDE HEIGHT: 20mm

CAMBER: -1.5 °    TOE: 1 °     □IN/     ■OUT

CAMBER LINK:  INSIDE 1 WASHERS 1 OUTSIDE A WASHERS 0

                                    1=inside                                               A=inside

CASTER BLOCK:    □20°         ■25°(stock)        □30°

                           □BACK         ■FORWARD (stock)

BUMP STEER SPACERS: spacer nylon □.050”(stock/ ■OTHER: _4mm

(spacers under outer tie-rod ballstud)

AXLE:    □TRAILING(stock)/   ■INLINE

AXLE HEIGHT:    □TOP         □MIDDLE         ■BOTTOM

STEERING ACKERMAN:  ■ FORWARD(stock)         □BACK


	SHOCKS

■AE/    □LOSI

MOUTING(T/B): 2 /2 

                                         1=inside

SPRING: Brown_

PISTON: 2 

      ■STANDARD/      □DRILLED

OIL: 30 LOSI

LIMITING(T/B): stock

	REAR

	SUSPENSION

TRANSMISSION:    ■4 Gear(Stock)/    □3 Gear 

TRANSMISSION HEIGHT:  .60(stock)/    ■OTHER: silver 0.20

RIDE HEIGHT: 23mm_

CAMBER:_-1.5_ °     TOE BAR:    ■3°  □4°  ANTISQUAT: Gold 0.60

CAMBER LINK:  INSIDE: 1 WASHERS: 0   OUTSIDE: B 

                                                            1=inside                                                       A=inside

HUB SPACING:    ■FORWARD         □MIDDLE         □BACK

ARM SPACING:    ■FORWARD         □MIDDLE         □BACK      
	SHOCKS

■AE/    □LOSI

MOUTING(T/B): 2_ /_2

                                         1=inside

SPRING:_Silver___

PISTON:_2_

       ■STANDARD/     □DRILLED

OIL:_25 LOSI

LIMITING(T/B): stock



	OTHER

	* Weight behind servo 60g, Front of servo 10g

* Low profile steering servo: futaba 9551.

* Front kick-up lowered 5° → thats why 25° caster

* Try rear camber link inside 2 outside C

* Kinwald front hub mod → Front hub moved 2mm forward



	FRONT TIRES: JC rounder
	FRONT INSERTS: JC

	REAR TIRES: HoleShot 2.0
	REAR INSERTS: Proline

	ESC: CS Electronics
	MOTOR: CS 7,5t

	BATTERY: LiPo Robitronic 5000mah
	PINION/SPUR: 21 / 78

	WING: X-factory, forward and low, cut 10mm


